----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 09.11.2025 11:09:49

-- Design Name:

-- Module Name: ModN - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity ModN is

generic( N : integer := 10);

Port ( clk : in STD_LOGIC;

rst : in STD_LOGIC;

counter : out STD_LOGIC_VECTOR (3 downto 0));

end ModN;

architecture Behavioral of ModN is

signal cnt : integer range 0 to N - 1;

signal clkout   : STD_LOGIC;

signal count : integer range 0 to 49999999;

begin

process(clk, rst)

begin

if rst = '1' then

clkout <= '0';

count <= 0;

elsif (clk'event and clk = '1') then

if (count = 49999999) then

clkout <= not clkout;

count<= 0;

else

count <= count + 1;

end if;

end if;

end process;

process(clk,rst)

begin

if rst = '1' then

cnt <= 0;

elsif (clkout'event and clkout = '1') then

if cnt = N - 1 then

cnt <= 0;

else cnt <= cnt + 1;

end if;

end if;

end process;

counter <= std_logic_vector(TO_UNSIGNED(cnt, 7));

end Behavioral;