Experiment 5

Name: Neha D. Rane 	                                                                                                                                        Batch: B1

BE ENTC-1                                                                                                                                                                   VLSI LAB

Roll no: 2069											    2024-2025

- VHDL Code

PmodKYPD Entity

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity PmodKYPD is

Port (

clk : in STD_LOGIC;

JA : inout STD_LOGIC_VECTOR (7 downto 0);  -- PmodKYPD connected to JA

an : out STD_LOGIC_VECTOR (3 downto 0);    -- Controls which 7-segment display digit

seg : out STD_LOGIC_VECTOR (6 downto 0)    -- Display digits on the 7-segment display

);

end PmodKYPD;

architecture Behavioral of PmodKYPD is

component Decoder is

Port (

clk : in STD_LOGIC;

Row : in STD_LOGIC_VECTOR (3 downto 0);

Col : out STD_LOGIC_VECTOR (3 downto 0);

DecodeOut : out STD_LOGIC_VECTOR (3 downto 0)

);

end component;

component DisplayController is

Port (

DispVal : in STD_LOGIC_VECTOR (3 downto 0);

anode : out STD_LOGIC_VECTOR (3 downto 0);

segOut : out STD_LOGIC_VECTOR (6 downto 0)

);

end component;

signal Decode: STD_LOGIC_VECTOR (3 downto 0);

begin

C0: Decoder port map (

clk => clk,

Row => JA(7 downto 4),

Col => JA(3 downto 0),

DecodeOut => Decode

);

C1: DisplayController port map (

DispVal => Decode,

anode => an,

segOut => seg

);

end Behavioral;

Decoder Entity

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Decoder is

Port (

clk : in STD_LOGIC;

Row : in STD_LOGIC_VECTOR (3 downto 0);

Col : out STD_LOGIC_VECTOR (3 downto 0);

DecodeOut : out STD_LOGIC_VECTOR (3 downto 0)

);

end Decoder;

architecture Behavioral of Decoder is

signal sclk : STD_LOGIC_VECTOR(19 downto 0);

begin

process(clk)

begin

if clk'event and clk = '1' then

-- 1ms

if sclk = "00011000011010100000" then

-- C1

Col <= "0111";

sclk <= sclk + 1;

elsif sclk = "00011000011010101000" then

if Row = "0111" then

DecodeOut <= "0001"; -- 1

elsif Row = "1011" then

DecodeOut <= "0100"; -- 4

elsif Row = "1101" then

DecodeOut <= "0111"; -- 7

elsif Row = "1110" then

DecodeOut <= "0000"; -- 0

end if;

sclk <= sclk + 1;

elsif sclk = "00110000110101000000" then

-- C2

Col <= "1011";

sclk <= sclk + 1;

elsif sclk = "00110000110101001000" then

if Row = "0111" then

DecodeOut <= "0010"; -- 2

elsif Row = "1011" then

DecodeOut <= "0101"; -- 5

elsif Row = "1101" then

DecodeOut <= "1000"; -- 8

elsif Row = "1110" then

DecodeOut <= "1111"; -- F

end if;

sclk <= sclk + 1;

elsif sclk = "01001001001111100000" then

-- C3

Col <= "1101";

sclk <= sclk + 1;

elsif sclk = "01001001001111101000" then

if Row = "0111" then

DecodeOut <= "0011"; -- 3

elsif Row = "1011" then

DecodeOut <= "0110"; -- 6

elsif Row = "1101" then

DecodeOut <= "1001"; -- 9

elsif Row = "1110" then

DecodeOut <= "1110"; -- E

end if;

sclk <= sclk + 1;

elsif sclk = "01100001101010000000" then

-- C4

Col <= "1110";

sclk <= sclk + 1;

elsif sclk = "01100001101010001000" then

if Row = "0111" then

DecodeOut <= "1010"; -- A

elsif Row = "1011" then

DecodeOut <= "1011"; -- B

elsif Row = "1101" then

DecodeOut <= "1100"; -- C

elsif Row = "1110" then

DecodeOut <= "1101"; -- D

end if;

sclk <= "00000000000000000000";

else

sclk <= sclk + 1;

end if;

end if;

end process;

end Behavioral;

DisplayController Entity

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity DisplayController is

Port (

-- Output from the Decoder

DispVal : in STD_LOGIC_VECTOR (3 downto 0);

-- Controls the display digits

anode : out STD_LOGIC_VECTOR (3 downto 0);

-- Controls which digit to display

segOut : out STD_LOGIC_VECTOR (6 downto 0)

);

end DisplayController;

architecture Behavioral of DisplayController is

begin

-- Only display the leftmost digit

anode <= "1110";

with DispVal select

segOut <= "1000000" when "0000",  -- 0

"1111001" when "0001",  -- 1

"0100100" when "0010",  -- 2

"0110000" when "0011",  -- 3

"0011001" when "0100",  -- 4

"0010010" when "0101",  -- 5

"0000010" when "0110",  -- 6

"1111000" when "0111",  -- 7

"0000000" when "1000",  -- 8

"0010000" when "1001",  -- 9

"0001000" when "1010",  -- A

"0000011" when "1011",  -- B

"1000110" when "1100",  -- C

"0100001" when "1101",  -- D

"0000110" when "1110",  -- E

"0001110" when "1111",  -- F

"0111111" when others;

end Behavioral;

- Synthesis Report

Detailed RTL Component Info :

+---Adders :

2 Input   20 Bit       Adders := 1

+---Registers :

20 Bit    Registers := 1

4 Bit    Registers := 2

+---Muxes :

2 Input   20 Bit        Muxes := 1

2 Input    4 Bit        Muxes := 8

4 Input    4 Bit        Muxes := 3

2 Input    1 Bit        Muxes := 3

Report Cell Usage:

+------+-------+------+

|      |Cell   |Count |

+------+-------+------+

|1     |BUFG   |     1|

|2     |CARRY4 |     5|

|3     |LUT2   |    11|

|4     |LUT3   |     5|

|5     |LUT4   |    22|

|6     |LUT5   |    39|

|7     |LUT6   |    46|

|8     |FDRE   |    28|

|9     |IBUF   |     5|

|10    |OBUF   |    15|

+------+-------+------+

Report Instance Areas:

+------+---------+------------------+------+

|      |Instance |Module            |Cells |

+------+---------+------------------+------+

|1     |top      |                  |   177|

|2     |  C0     |Decoder           |   149|

|3     |  C1     |DisplayController |     7|

+------+---------+------------------+------+

- UCF file

set_property -dict { PACKAGE_PIN W7   IOSTANDARD LVCMOS33 } [get_ports {seg[0]}]

set_property -dict { PACKAGE_PIN W6   IOSTANDARD LVCMOS33 } [get_ports {seg[1]}]

set_property -dict { PACKAGE_PIN U8   IOSTANDARD LVCMOS33 } [get_ports {seg[2]}]

set_property -dict { PACKAGE_PIN V8   IOSTANDARD LVCMOS33 } [get_ports {seg[3]}]

set_property -dict { PACKAGE_PIN U5   IOSTANDARD LVCMOS33 } [get_ports {seg[4]}]

set_property -dict { PACKAGE_PIN V5   IOSTANDARD LVCMOS33 } [get_ports {seg[5]}]

set_property -dict { PACKAGE_PIN U7   IOSTANDARD LVCMOS33 } [get_ports {seg[6]}]

#set_property -dict { PACKAGE_PIN V7   IOSTANDARD LVCMOS33 } [get_ports dp]

set_property -dict { PACKAGE_PIN U2   IOSTANDARD LVCMOS33 } [get_ports {an[0]}]

set_property -dict { PACKAGE_PIN U4   IOSTANDARD LVCMOS33 } [get_ports {an[1]}]

set_property -dict { PACKAGE_PIN V4   IOSTANDARD LVCMOS33 } [get_ports {an[2]}]

set_property -dict { PACKAGE_PIN W4   IOSTANDARD LVCMOS33 } [get_ports {an[3]}]

##Pmod Header JA

set_property -dict { PACKAGE_PIN J1   IOSTANDARD LVCMOS33 } [get_ports {JA[0]}];#Sch name = JA1

set_property -dict { PACKAGE_PIN L2   IOSTANDARD LVCMOS33 } [get_ports {JA[1]}];#Sch name = JA2

set_property -dict { PACKAGE_PIN J2   IOSTANDARD LVCMOS33 } [get_ports {JA[2]}];#Sch name = JA3

set_property -dict { PACKAGE_PIN G2   IOSTANDARD LVCMOS33 } [get_ports {JA[3]}];#Sch name = JA4

set_property -dict { PACKAGE_PIN H1   IOSTANDARD LVCMOS33 } [get_ports {JA[4]}];#Sch name = JA7

set_property -dict { PACKAGE_PIN K2   IOSTANDARD LVCMOS33 } [get_ports {JA[5]}];#Sch name = JA8

set_property -dict { PACKAGE_PIN H2   IOSTANDARD LVCMOS33 } [get_ports {JA[6]}];#Sch name = JA9

set_property -dict { PACKAGE_PIN G3   IOSTANDARD LVCMOS33 } [get_ports {JA[7]}];#Sch name = JA10

set_property -dict { PACKAGE_PIN W5   IOSTANDARD LVCMOS33 } [get_ports clk]