FIFO

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FIFO is

Port ( clk : in STD_LOGIC;

rst : in STD_LOGIC;

RD_EN : in STD_LOGIC;

WR_EN : in STD_LOGIC;

data_in : in STD_LOGIC_VECTOR (7 downto 0);

full : out STD_LOGIC;

empty : out STD_LOGIC;

data_out : out STD_LOGIC_VECTOR (7 downto 0));

end FIFO;

architecture Behavioral of FIFO is

type FIFO_IS_ARRAY is array (0 to 3) of std_logic_vector (7 downto 0);

signal memory : FIFO_IS_ARRAY;

signal rdptr, wrptr : integer range 0 to 3;

type data_is_array is array (0 to 9) of std_logic_vector (7 downto 0);

signal data : data_is_array := ("00111111", "11100000", "11011100", "11001100",

"10101010", "11111000", "11110101", "11000000");

signal clkout : Std_Logic;

signal counter : integer range 0 to 99999999;

begin

process(clk, rst)

begin

if rst = '1' then

clkout <= '0';

counter <= 0;

elsif (clk'event and clk = '1') then

if counter = 99999999 then

clkout <= not clkout;

counter <= 0;

else

counter <= counter + 1;

end if;

end if;

end process;

process(clkout, rst)

begin

if (clkout'event and clkout = '1') then

if rst = '1' then

for i in 0 to 3 loop

memory(i) <= "10000000";

end loop;

empty <= '1';

full <= '0';

rdptr <= 0;

wrptr <= 0;

end if;

if rst /= '1' then

-- WRITE process

if (WR_EN = '1') then

if (wrptr < 4) then

memory(wrptr) <= data_in;

empty <= '0';

if (wrptr = 3) then

full <= '1';

wrptr <= wrptr + 1;

else

wrptr <= wrptr + 1;

end if;

end if;

end if;

-- READ process

if (RD_EN = '1') then

if (rdptr < 4) then

data_out <= memory(rdptr);

if (rdptr = 3) then

empty <= '1';

end if;

rdptr <= rdptr + 1;

end if;

end if;

end if;

end if;

end process;

end Behavioral;